G0JKN ShackSwitch

I2C Bus Architecture & Expansion Plan

Design Sketch — March 2026

# Overview

This document describes the I2C bus architecture for the G0JKN ShackSwitch — an open source Arduino-based HF shack controller. The design uses MCP23017 I2C GPIO expander chips to extend the Arduino Uno R4 WiFi's native GPIO, enabling control of a KK1L 2x6 relay matrix antenna switch, hardware configuration via jumpers, shack power sequencing, and future expansion to PA protection, BCD band decoding, and Radioberry integration.

The entire expansion chain runs on just two wires from the Arduino (SDA and SCL), keeping wiring minimal and the system modular. Each MCP23017 is address-configured via three hardware pins, allowing up to eight devices on a single bus.

# I2C Bus Map

| Address | Device | Status | Function |
| --- | --- | --- | --- |
| 0x20 | MCP23017 #1 | Build now | Relay matrix control (KK1L 2x6) |
| 0x21 | MCP23017 #2 | Build now | Hardware config jumpers (board type, inputs, radio) |
| 0x22 | MCP23017 #3 | Roadmap | Shack switching — amps, lights, PSU sequencing |
| 0x23-0x27 | TBD | Roadmap | Radioberry, BCD band decoder, PA protection |

# MCP23017 #1 — Relay Matrix Control (0x20)

Drives the KK1L 2x6 relay board. GPA controls Radio 1 outputs, GPB controls Radio 2 outputs. Two spare pins on GPB are reserved for future expansion.

| Pin | Direction | Function |
| --- | --- | --- |
| GPA0 | Output | Radio 1 — Antenna port 1 |
| GPA1 | Output | Radio 1 — Antenna port 2 |
| GPA2 | Output | Radio 1 — Antenna port 3 |
| GPA3 | Output | Radio 1 — Antenna port 4 |
| GPA4 | Output | Radio 1 — Antenna port 5 |
| GPA5 | Output | Radio 1 — Antenna port 6 |
| GPA6-7 | — | Spare |
| GPB0 | Output | Radio 2 — Antenna port 1 |
| GPB1 | Output | Radio 2 — Antenna port 2 |
| GPB2 | Output | Radio 2 — Antenna port 3 |
| GPB3 | Output | Radio 2 — Antenna port 4 |
| GPB4 | Output | Radio 2 — Antenna port 5 |
| GPB5 | Output | Radio 2 — Antenna port 6 |
| GPB6-7 | — | Spare (future expansion) |

The KK1L 2x6 board hardware enforces that no two radios can share the same antenna port. The firmware must also enforce this in software before sending any switch command.

# MCP23017 #2 — Hardware Config Jumpers (0x21)

Read on every boot and after factory reset. GPA pins are wired to jumpers or a DIP switch on the PCB. The firmware reads the byte and derives its complete operating configuration — no reflashing or web configuration needed for basic hardware setup.

| Bits | Field | Values |
| --- | --- | --- |
| GPA 0-1 | Board type | 00 = 4-port original ¦ 01 = KK1L 2x6 ¦ 10 = KK1L 2x8 ¦ 11 = future |
| GPA 2-3 | Num inputs | 00 = 1 radio ¦ 01 = 2 radios ¦ 10 = 4 radios |
| GPA 4-5 | Radio type | 00 = FlexRadio ¦ 01 = Generic CAT ¦ 10 = BCD decoder |
| GPA 6 | PA present | 0 = no PA ¦ 1 = PA fitted |
| GPA 7 | Spare | Reserved for future use |
| GPB 0-7 | Spare | Reserved — all inputs, pulled high |

Boot sequence — config detection

Wire.begin();

Wire.beginTransmission(0x21);          // probe config chip

bool configFound = (Wire.endTransmission() == 0);

if (configFound) {

uint8_t cfg = mcp2.readGPIOA();      // read jumper byte

boardType   = (cfg >> 0) & 0x03;

numInputs   = (cfg >> 2) & 0x03;

radioType   = (cfg >> 4) & 0x03;

paPresent   = (cfg >> 6) & 0x01;

} else {

// Fall back to EEPROM stored config or defaults

}

# MCP23017 #3 — Shack Switching (0x22) — Roadmap

Handles station power sequencing, PA interlock, and shack accessories. Critically, GPB pins are inputs — the firmware reads fault and protection signals back from hardware, giving closed-loop protection rather than blind switching.

| Pin | Direction | Function |
| --- | --- | --- |
| GPA0 | Output | PA power relay (12V feed or SSR input) |
| GPA1 | Output | PA TX enable / PTT interlock |
| GPA2 | Output | ATU power |
| GPA3 | Output | Preamp / RX antenna relay |
| GPA4 | Output | Shack master power (via SSR or contactor) |
| GPA5 | Output | Cooling fan |
| GPA6 | Output | Shack lighting |
| GPA7 | — | Spare |
| GPB0 | Input | PA fault (read back from amp) |
| GPB1 | Input | SWR trip input (from protection circuit) |
| GPB2 | Input | Temperature trip input |
| GPB3 | Input | Antenna relay confirmed (read back from MCP23017 #1) |
| GPB4-7 | — | Spare / future protection inputs |

TX safety interlock

bool safeTX() {

bool antConfirmed = mcp3.digitalRead(GPB3);

bool paFaultClear = !mcp3.digitalRead(GPB0);

bool swrOK        = !mcp3.digitalRead(GPB1);

bool tempOK       = !mcp3.digitalRead(GPB2);

return antConfirmed && paFaultClear && swrOK && tempOK;

}

Shack-on power sequence

The following sequence must be followed in order to safely bring the station on air:

- Assert shack master power (GPA4)

- Wait 2 seconds — PSUs stabilise

- Assert PA power relay (GPA0)

- Wait 3 seconds — PA bias settles

- Assert PA TX enable (GPA1)

- Update Nextion display: Station Ready

The shack-off sequence reverses this order. The PA is always powered down before the PSU.

For mains voltage switching (shack master power), use a solid state relay (SSR) driven by GPA4. The MCP23017 GPIO is 3.3V / 25mA — sufficient for an SSR logic input but not for direct relay coil drive. For 12V relay coils, use a transistor driver stage (BC547 or similar) as used on the existing ShackSwitch shield.

# Extended /status JSON

The web server status endpoint extends to report all subsystem states. AetherSDR and Node-RED poll this endpoint and adapt their UI based on the reported topology:

{"inputs":2,"outputs":6,

"radio1":2,"radio2":4,

"pa_power":true,"pa_fault":false,

"swr_trip":false,"temp_ok":true,

"shack_on":true,"fan":false}

# KK1L 2x6 Relay Board — Key Specs

| Parameter | Value |
| --- | --- |
| Price | $33.00 (kk1l.com) |
| PCB | FR4 double-sided, 2oz copper |
| Power rating | 1500W+ (2800W key-down into 50 ohms) |
| Port-to-port isolation | >61dB |
| Relay activation | 12V DC |
| Unused port termination | 50 ohms (reduces reradiation and static) |
| Safety | Impossible to connect both radios to same port |
| Control lines required | 12 total (6 per radio input) |
| Dimensions | 10.1875" x 6.5" |

# References & Links

- ShackSwitch firmware: https://github.com/nigelfenton/shackswitch

- AetherSDR project: https://github.com/ten9876/AetherSDR

- AetherSDR issue #179 (antenna switch integration): https://github.com/ten9876/AetherSDR/issues/179

- KK1L 2x6 relay board: https://kk1l.com/store/2x6-relay-board/

- MCP23017 datasheet: https://www.microchip.com/en-us/product/mcp23017

G0JKN ShackSwitch — Design Sketch, March 2026. Open source under MIT Licence.