G0JKN ShackSwitch

Project Brief — Version 1.5

Nigel Fenton, G0JKN

Last updated: 27 March 2026

# 1. Project Overview

The G0JKN ShackSwitch is an open-source Arduino-based HF shack controller and antenna switcher, designed and built by Nigel Fenton (G0JKN), a retired amateur radio operator based in the UK.

The device provides local touchscreen control, web-based remote control, a TCP control protocol, UDP network discovery, SO2R interlock logic, and FlexRadio band tracking. It is designed to be hardware-configurable without firmware reflashing, using I2C GPIO expanders for board detection at boot.

GitHub repository: https://github.com/nigelfenton/shackswitch

# 2. Current Hardware (Built and Verified)

| Component | Detail |
| --- | --- |
| Microcontroller | Arduino Uno R4 WiFi — main controller |
| Touchscreen | Nextion NX4832T035 3.5" colour display — local control via EasyNextionLibrary |
| Relay module | 4x 5V relay on pins D2–D5 — antenna switching |
| Shield | Custom Arduino shield with relays, transistor drivers and connectors |
| RF connectors | 5x SO239 — one radio input, four antenna outputs |
| Power | 12V DC supply, buck converter 12V→5V |
| Enclosure | 3D printed, designed in Fusion 360 |
| Firmware | v1.5 — see Section 4 for full feature list |

# 3. Expansion Hardware (Ordered, Not Yet Built)

The following hardware has been ordered and will be integrated in the next build phase:

- KK1L 2×6 relay board — upgrades from 4-port to 2-input 6-output matrix. 1500W+, 61dB isolation, 12V relays, 12 control lines required. Source: kk1l.com

- MCP23017 I2C GPIO expanders — to drive the KK1L board and provide hardware configuration detection

# 4. Firmware v1.5 — Feature Summary

The current firmware is v1.5, a significant advance on the v1.4 described in earlier documentation. All features below are implemented and verified.

## 4.1 Web Server (Port 80)

- Relay control via browser UI — /[n]/on and /[n]/off

- Antenna rename — persists to EEPROM, updates Nextion and web page

- Status endpoint — GET /status returns relay states, band data and SO2R status

- Band update endpoint — GET /setband?input=[1|2]&band=[name] sets band for Input 1 or Input 2

- Settings page — WiFi SSID/password configuration

## 4.2 TCP Control Protocol (Port 9008)

- Line-based protocol: commands prefixed C[seq]|, responses prefixed R[seq]| or S0|

- Commands: ping, antenna list, band list, port get, port set, sub port all, interlock set

- Designed for AetherSDR integration and future Node-RED TCP control

- Keepalive — S0|ping sent every 30 seconds to connected client

## 4.3 UDP Discovery Beacon

- Broadcasts device identity every 5 seconds on port 9008

- Payload includes device name, serial, version, IP address, port count and radio count

- Enables automatic discovery by AetherSDR and other network clients

## 4.4 FlexRadio Band Tracking

- portA and portB structures track band, antenna selection, TX state and inhibit status independently

- Band defined by ID (1–11 covering 160m–6m) with frequency range lookup via bandForFreq()

- Band name reverse lookup via band name string → ID for REST /setband endpoint

- Band display updated on Nextion via tBandA, tBandB, tSO2R components

## 4.5 SO2R Interlock

- Evaluates interlock whenever TX state or band changes

- Inhibits Radio B if both radios are transmitting on the same band or same antenna

- Nextion tSO2R shows SO2R OK (green) or INHIBIT (orange)

- Interlock state included in /status REST response and TCP port status push

## 4.6 Other Features

- NTP time sync → RTC, displayed on Nextion tClock

- WiFi RSSI monitoring on Nextion monitor page

- Background WiFi reconnect every 30 seconds if connection lost

- Factory reset with double-tap confirmation and 5-second safety timeout

- LED matrix displays active relay number or ground symbol

# 5. REST API Endpoints

| Endpoint | Description |
| --- | --- |
| GET /status | Returns relay states, bandA, bandB, so2r as JSON |
| GET /[n]/on | Activate relay n (1–4), ground all others |
| GET /[n]/off | Ground relay n |
| GET /rename?id=[n]&name=[name] | Rename antenna port n, persist to EEPROM |
| GET /setband?input=[1¦2]&band=[name] | Set band for Input 1 or 2 by name (e.g. 40m). Updates Nextion and evaluates SO2R interlock. |
| GET /settings | Settings web page |

# 6. Input Port Architecture

The ShackSwitch is architecturally a two-input device. The two inputs are defined at the hardware level by the KK1L 2×6 relay matrix (2 inputs, 6 outputs) and reflected in the firmware port model.

Input 1 and Input 2 are the correct terms throughout firmware, API and documentation. What is connected to each input is configuration, not definition:

- Current setup: Input 1 = FlexRadio Slice A, Input 2 = FlexRadio Slice B

- Alternative: Input 1 = Radio A, Input 2 = Radio B (two separate transceivers)

The FlexRadio 6000 series can present multiple receive slices simultaneously, each potentially on a different band, with TX active on only one slice at a time. For antenna switching purposes the TX slice is the safety-critical path. The Node-RED integration should subscribe to TX slice identity from SmartSDR, not just band change events.

Note: Multi-RX within a single slice (up to 4 RX, 2 per slice) is a known complexity. RX-only paths do not require independent antenna switching. This is logged as a design consideration for the Node-RED flow and PA protection logic.

# 7. I2C Bus Architecture (Planned)

| Address | Device | Purpose |
| --- | --- | --- |
| 0x20 | MCP23017 #1 | KK1L 2×6 relay matrix control |
| 0x21 | MCP23017 #2 | Hardware config jumpers (board type, inputs, radio type, PA present) |
| 0x22 | MCP23017 #3 | Shack switching — amps, lights, PSU sequencing (roadmap) |
| 0x23–0x27 | TBD | Radioberry, BCD decoder, PA protection (roadmap) |

## 7.1 MCP23017 #1 Pin Map (KK1L Control)

Pin numbering convention: pin number always equals port number. GPA0 and GPB0 are deliberately spare. Firmware uses mask = (1 << port) — unambiguous and clean.

| Pin | Assignment |
| --- | --- |
| GPA0 | Spare |
| GPA1–6 | Input 1 (Slice A) — Antenna ports 1–6 |
| GPA7 | Spare |
| GPB0 | Spare |
| GPB1–6 | Input 2 (Slice B) — Antenna ports 1–6 |
| GPB7 | Spare |

## 7.2 MCP23017 #2 Config Bits

| Bits | Values | Meaning |
| --- | --- | --- |
| 0–1 | 00=4-port, 01=2×6, 10=2×8, 11=future | Board type |
| 2–3 | 00=1, 01=2, 10=4 | Number of inputs |
| 4–5 | 00=Flex, 01=generic CAT, 10=BCD | Radio type |
| 6 | 0/1 | PA present |
| 7 | — | Spare |

Boot detection: firmware probes 0x21 at startup, reads config byte, derives operating mode. Falls back to EEPROM defaults if chip not found. No reflashing required when changing hardware.

# 8. Related Project — AetherSDR

AetherSDR (https://github.com/ten9876/AetherSDR) is a Linux-native Qt6/C++20 FlexRadio client. Nigel raised issue #179 proposing a native antenna switch integration panel that communicates with the ShackSwitch REST API and TCP protocol.

Integration points:

- REST /status — panel polls for relay state, band and SO2R status

- REST /[n]/on and /[n]/off — panel drives relay selection

- TCP port 9008 — richer integration via port set and interlock set commands

- UDP discovery — AetherSDR can auto-discover ShackSwitch on the local network

The AetherSDR panel should label inputs as Input 1 / Input 2 (or user-renameable) rather than hard-coding Radio 1 / Radio 2, to support both single-radio dual-slice and two-radio configurations.

# 9. Node-RED Integration (Planned)

Node-RED running on a Raspberry Pi will bridge FlexRadio SmartSDR frequency data to the ShackSwitch, enabling automatic band-based antenna switching.

Requirements documented, implementation not yet started:

- Subscribe to TX slice identity and band change events from SmartSDR

- Determine which input port corresponds to the active TX slice

- Call GET /setband?input=[1|2]&band=[name] on band change

- Debounce rapid VFO changes to avoid relay chatter

- Handle radio startup, disconnect and split operation gracefully

- Do not switch antennas on RX-only slice band changes without TX slice confirmation

# 10. Nextion HMI Components

The following components are defined on the Nextion display. Label widths for antenna names (t3–t6) require adjustment in the Nextion editor to accommodate longer names up to 25 characters.

| Component | Purpose |
| --- | --- |
| b1–b4 | Dual-state antenna selector buttons (page 0) |
| t3–t6 | Antenna name labels (t3=b1, t4=b2, t5=b3, t6=b4) |
| tBandA | Current band for Input 1 / Slice A |
| tBandB | Current band for Input 2 / Slice B |
| tSO2R | SO2R status — SO2R OK (green) or INHIBIT (orange) |
| tState | Antenna state — ANT Active or ANT Grounded |
| tClock | RTC time display |
| t1 | WiFi IP address |
| tRSSI | WiFi signal strength in dBm (monitor page) |
| nSignal | WiFi signal quality bar (monitor page) |
| b0 | IP address button (monitor page) |

# 11. Project Documents

- shackswitch_project_brief.docx — this document

- shackswitch_i2c_architecture.docx — full I2C bus map and pin tables

- shackswitch_test_plan.docx — build and commissioning test plan (covers original 4-port build; update required for v1.5 and KK1L expansion)

# 12. Roadmap

| Priority | Item | Notes |
| --- | --- | --- |
| Immediate | Nextion label widths | Increase t3–t6 component width in HMI editor |
| Near term | KK1L board build | Hardware ordered. Requires transistor/ULN2803 driver stage for 12V relays from MCP23017 outputs. |
| Near term | MCP23017 integration | Firmware support for 0x20 relay matrix and 0x21 boot config detection |
| Near term | Node-RED flow | SmartSDR → /setband integration. TX slice tracking essential. |
| Near term | Test plan update | Update shackswitch_test_plan.docx for v1.5 features and KK1L build |
| Near term | AetherSDR #179 | Confirm API sufficiency for 2-input matrix. Neutral input labelling in panel. |
| Roadmap | MCP23017 #3 | Shack switching — amps, lights, PSU sequencing |
| Roadmap | PA protection | TX slice tracking feeds into sequencer logic. Must not fire PA on wrong antenna. |
| Roadmap | Multi-RX handling | FlexRadio multi-RX per slice. RX-only paths do not drive switching. TX slice is master. |
| Roadmap | Radioberry / BCD | 0x23+ on I2C bus, TBD |