G0JKN ShackSwitch

Build & Commissioning Test Plan

Configuration: Original 4-port build | Arduino Uno R4 WiFi | Nextion 3.5"

Version: 1.5  |  March 2026  |  Firmware v1.5

Nigel Fenton, G0JKN

# 1. Purpose

This test plan provides a structured, repeatable procedure for commissioning a G0JKN ShackSwitch from first power-on through to full RF verification. It covers the original 4-port build and all v1.5 firmware features including the TCP control protocol, UDP discovery beacon, band tracking, SO2R interlock, and REST API extensions.

Section 13 covers the KK1L 2x6 expansion build, to be completed once hardware arrives.

Each test section is independent. Work through them in order. Do not proceed to a later section if earlier tests have unresolved failures — the sections build on each other.

# 2. Equipment & Tools Required

| Item | Detail / Specification |
| --- | --- |
| Multimeter | DC voltage and continuity / resistance ranges |
| 12V DC power supply | Minimum 2A — bench PSU preferred for first power-on |
| USB cable | USB-A to USB-B for Arduino programming |
| PC with Arduino IDE | Version 2.x, R4 board package installed |
| PC or phone browser | On the same WiFi network as the ShackSwitch |
| Terminal / netcat | For TCP protocol tests on port 9008 (e.g. PuTTY raw mode, nc, or netcat) |
| Coax jumpers x5 | SO239 / PL259 — one for radio input, four for antenna outputs |
| Dummy load x1 (min) | 50 ohm, rated for your transceiver power level |
| Transceiver (low power) | Any HF radio — 5W is sufficient for RF path tests |
| SWR / power meter | Inline between transceiver and ShackSwitch |
| Small flat screwdriver | For checking connector screws and shield mounts |

WARNING: Do not apply RF power during any electrical or relay test. RF path testing is Section 12 only, and only after all earlier tests have passed.

# 3. Safety Notes

- The ShackSwitch operates from 12V DC. Short circuits can cause fire, damage components, or destroy the Arduino. Always verify wiring before applying power.

- Disconnect 12V power before making any wiring changes or seating/removing the Arduino from the shield.

- RF power is only applied in Section 12. Keep power at 5W or below during initial RF testing.

- If at any point you smell burning or see smoke — disconnect power immediately. Do not re-apply until the fault is found and corrected.

# 4. Pre-Power Visual Inspection

Complete this section before connecting any power. The goal is to catch obvious wiring errors, shorts, and assembly problems before they can cause damage.

| # | Action / Step | Expected Result | Pass | Fail | Notes / If Failed |
| --- | --- | --- | --- | --- | --- |
| 4.1 Arduino & Shield |  |  |  |  |  |
| 01 | Inspect the Arduino Uno R4 WiFi — confirm it is correctly seated in the shield headers with no bent pins. | All pins engaged. No pins folded under the board. |  |  | Re-seat if needed. Check for bent pins with a magnifier. |
| 02 | Inspect relay module — confirm all relay coil connections to shield pins D2, D3, D4, D5 are secure. | Four relay connections present and firm. No bare wire strands touching adjacent pins. |  |  | Re-seat any loose connectors. |
| 03 | Inspect the Nextion display connector — confirm TX1/RX1 and 5V/GND wiring to the display. | Four wires: TX1→RX, RX1→TX, 5V, GND. No crossed TX/RX wires. |  |  | TX/RX crossed is the most common Nextion wiring error — swap if display is blank. |
| 04 | Visually inspect the PCB/veroboard for solder bridges, especially around relay driver transistors and shield header pins. | No visible solder bridges. Joints are clean and shiny. |  |  | Use solder wick to remove any bridges. Re-inspect. |
| 4.2 Power Wiring |  |  |  |  |  |
| 05 | Identify the 12V DC input connector. Confirm polarity markings match the power supply — positive centre (barrel jack) or XT60 correct orientation. | Polarity confirmed correct before any connection is made. |  |  | Do not proceed until polarity is confirmed. Reverse polarity will destroy components. |
| 06 | Check the buck converter output wiring (12V in → 5V out → Arduino 5V pin). Measure the buck converter output with a multimeter before connecting to Arduino. | Buck converter output measures 4.9V–5.1V. |  |  | Adjust buck converter trimmer pot until output is within range. |
| 4.3 RF Connectors |  |  |  |  |  |
| 07 | Confirm all five SO239 connectors are present — one labelled RADIO IN and four labelled ANT 1–4. Check they are firmly mounted. | Five connectors present and mechanically secure. Centre pins present in all five. |  |  | Tighten retaining nuts if any are loose. |
| 08 | Using multimeter in continuity mode — with 12V disconnected and no coax connected — check RADIO IN centre pin does NOT show continuity to any ANT 1–4 centre pin. | No continuity between RADIO IN and any antenna port (all relays open / grounded at rest). |  |  | Unexpected continuity indicates a relay or wiring fault. Investigate before powering on. |

# 5. First Power-On — Electrical

Apply 12V power for the first time in this section. Have your finger near the power switch or connector so you can disconnect immediately if anything is wrong.

| # | Action / Step | Expected Result | Pass | Fail | Notes / If Failed |
| --- | --- | --- | --- | --- | --- |
| 5.1 Power Rails |  |  |  |  |  |
| 01 | Connect 12V supply. Observe the board for 5 seconds before doing anything else. | No smoke, no burning smell, no components becoming hot to the touch. |  |  | If any of these occur — disconnect immediately and identify the fault before proceeding. |
| 02 | Measure voltage at the Arduino 5V pin (or shield 5V rail) with multimeter. | Reads 4.9V–5.1V. |  |  | If low (<4.8V) — check buck converter output and wiring. If zero — check buck converter power input. |
| 03 | Measure voltage across the 12V relay coil supply on the shield. | Reads 11.5V–12.5V. |  |  | If zero — check 12V input wiring to the shield. |
| 5.2 Power LED |  |  |  |  |  |
| 04 | Observe the blue power LED on the back panel. | Blue LED illuminated steady. |  |  | If not lit — check LED polarity and current limiting resistor. |
| 5.3 Arduino Boot |  |  |  |  |  |
| 05 | Observe the Arduino onboard LED matrix during boot (first 5 seconds after power-on). | LED matrix shows the boot pattern / relay state display. Matrix illuminates without error. |  |  | If matrix is blank — check 5V supply to Arduino. If garbled — re-upload firmware. |

# 6. Nextion Display

The Nextion display must be running the correct HMI file. If the display is blank or showing factory default content, upload the ShackSwitch HMI file via microSD card before proceeding.

| # | Action / Step | Expected Result | Pass | Fail | Notes / If Failed |
| --- | --- | --- | --- | --- | --- |
| 6.1 Display Boot |  |  |  |  |  |
| 01 | Power on and observe the Nextion display. | Display illuminates and shows Page 0 — four antenna port buttons (all grounded state), band display area, SO2R status, clock area, and IP address field. |  |  | If blank — check 5V and GND to display. If showing wrong content — re-upload HMI file. |
| 02 | Observe the button states on the Nextion at boot. | All four antenna buttons show grounded (inactive) state. No button shows active/green. |  |  | If any button shows active — check syncButtonStates() call in setup(). Re-upload firmware. |
| 6.2 Antenna Labels |  |  |  |  |  |
| 03 | Observe text labels above each of the four antenna buttons. | Labels t3–t6 are visible above the buttons. Default names or previously saved names displayed. Text not clipped. |  |  | If labels blank — confirm HMI component names match firmware (t3, t4, t5, t6). If clipped — increase component width in Nextion editor. |
| 6.3 Band Display |  |  |  |  |  |
| 04 | Observe tBandA, tBandB and tSO2R components on the display at boot. | tBandA shows '---', tBandB shows '---'. tSO2R shows 'SO2R OK' in green text. |  |  | If tSO2R is black or blank — check updateNextionBandDisplay() is called in setup(). Check tSO2R component name in HMI. |
| 6.4 NTP Clock |  |  |  |  |  |
| 05 | Wait up to 60 seconds after WiFi connects (see Section 7). Observe the clock display on the Nextion. | Clock shows current UTC time and updates each minute. |  |  | If clock shows 00:00 — check WiFi connection (Section 7). NTP requires internet access. |

# 7. WiFi & Network

The ShackSwitch must connect to your local WiFi network. If WiFi credentials are not yet configured, either edit loadConfig() and re-upload, or use the Nextion config page to enter credentials.

| # | Action / Step | Expected Result | Pass | Fail | Notes / If Failed |
| --- | --- | --- | --- | --- | --- |
| 7.1 WiFi Connection |  |  |  |  |  |
| 01 | Power on and wait up to 60 seconds. Observe the IP address field on the Nextion display. | An IP address in your local network range is displayed (e.g. 10.0.0.x). Not 0.0.0.0. |  |  | If shows N O T C O N N E C T E D — check SSID and password in firmware or via Nextion config page. |
| 02 | Note the IP address shown on the display. On a PC on the same network, open a browser and navigate to http://[IP address]. | The ShackSwitch web interface loads showing four antenna port cards with names and current state. |  |  | If page does not load — confirm device is on same network. Try pinging the IP address. |
| 7.2 Status Endpoint |  |  |  |  |  |
| 03 | In the browser, navigate to http://[IP address]/status | JSON response displayed: {"r1":0,"r2":0,"r3":0,"r4":0,"bandA":"---","bandB":"---","so2r":0} — all relays grounded at boot. |  |  | If page not found — check firmware web server routes. Re-upload firmware if needed. |
| 7.3 Settings Page |  |  |  |  |  |
| 04 | Navigate to http://[IP address]/settings | Settings page loads showing four antenna name input fields and Save buttons. |  |  | If page not found — check /settings route in firmware. |

# 8. Relay Switching — Touchscreen

This section verifies that the Nextion touchscreen correctly activates each relay and grounds all others. Use a multimeter in continuity mode between the RADIO IN centre pin and each ANT port centre pin to verify switching.

Set your multimeter to continuity/beep mode. Probe RADIO IN centre pin with one lead. Touch each ANT port centre pin with the other lead as you work through the tests.

| # | Action / Step | Expected Result | Pass | Fail | Notes / If Failed |
| --- | --- | --- | --- | --- | --- |
| 8.1 Antenna 1 |  |  |  |  |  |
| 01 | Press the Antenna 1 button on the Nextion display. | Button 1 changes to active (green) state. Buttons 2, 3, 4 remain grounded. |  |  | If display does not update — check trigger1() in triggers.ino. Re-upload firmware. |
| 02 | Check relay with multimeter — continuity between RADIO IN and ANT 1. | Continuity (beep) confirmed between RADIO IN and ANT 1. |  |  | If no continuity — check relay 1 wiring to D2. Check relay module power. |
| 03 | Check remaining ports — probe RADIO IN to ANT 2, ANT 3, ANT 4. | No continuity on ANT 2, ANT 3, ANT 4 (all grounded). |  |  | If continuity on any non-selected port — relay not releasing. Check relay driver circuit. |
| 04 | Check /status endpoint. | Returns {"r1":1,"r2":0,"r3":0,"r4":0} |  |  | If status does not match relay state — check relay state tracking in firmware. |
| 8.2 Antenna 2 |  |  |  |  |  |
| 05 | Press the Antenna 2 button on the Nextion display. | Button 2 active. Buttons 1, 3, 4 grounded. Continuity RADIO IN↔ANT 2 only. /status returns {"r1":0,"r2":1,"r3":0,"r4":0} |  |  | Relay not releasing — check driver transistor for that relay. |
| 06 | Verify ANT 1, 3, 4 show no continuity to RADIO IN. | No continuity on unselected ports. |  |  | Relay not releasing — check driver transistor for that relay. |
| 8.3 Antenna 3 |  |  |  |  |  |
| 07 | Press the Antenna 3 button on the Nextion display. | Button 3 active. Continuity RADIO IN↔ANT 3 only. /status returns {"r1":0,"r2":0,"r3":1,"r4":0} |  |  | Check trigger3() and relay 3 wiring to D4. |
| 08 | Verify ANT 1, 2, 4 show no continuity to RADIO IN. | No continuity on unselected ports. |  |  | Relay not releasing — check driver transistor for relay 3. |
| 8.4 Antenna 4 |  |  |  |  |  |
| 09 | Press the Antenna 4 button on the Nextion display. | Button 4 active. Continuity RADIO IN↔ANT 4 only. /status returns {"r1":0,"r2":0,"r3":0,"r4":1} |  |  | Check trigger4() and relay 4 wiring to D5. |
| 10 | Verify ANT 1, 2, 3 show no continuity to RADIO IN. | No continuity on unselected ports. |  |  | Relay not releasing — check driver transistor for relay 4. |
| 8.5 Single-Antenna Enforcement |  |  |  |  |  |
| 11 | With Antenna 2 active, rapidly press Antenna 4, then immediately press Antenna 1. | At no point are two antenna buttons shown as active simultaneously. Only the last pressed shows active. |  |  | If two relays ever close simultaneously — critical firmware fault. Do not use for RF until resolved. |

# 9. Relay Switching — Web Interface

Repeat the relay switching tests using the web browser interface. This confirms the web server, button handler, and relay control all work correctly from a network command, independent of the touchscreen.

| # | Action / Step | Expected Result | Pass | Fail | Notes / If Failed |
| --- | --- | --- | --- | --- | --- |
| 9.1 Web Switch — Each Port |  |  |  |  |  |
| 01 | In browser, click the Antenna 1 button on the web interface. | ANT 1 card highlighted green. ANT 2–4 shown grey. Multimeter confirms continuity RADIO IN↔ANT 1 only. |  |  | If click has no effect — check web server GET handler in firmware. |
| 02 | Click Antenna 2 button in browser. | ANT 2 active. All others grounded. Continuity confirmed RADIO IN↔ANT 2. |  |  | Check web handler for relay 2. |
| 03 | Click Antenna 3 button in browser. | ANT 3 active. All others grounded. Continuity confirmed RADIO IN↔ANT 3. |  |  | Check web handler for relay 3. |
| 04 | Click Antenna 4 button in browser. | ANT 4 active. All others grounded. Continuity confirmed RADIO IN↔ANT 4. |  |  | Check web handler for relay 4. |
| 9.2 Cross-Update — Web Follows Nextion |  |  |  |  |  |
| 05 | With ANT 3 active via web, switch to ANT 1 using the Nextion touchscreen. Wait 6 seconds then check the web interface. | Web interface updates to show ANT 1 active within 6 seconds without page refresh. |  |  | If web does not update — check /status polling JavaScript in web page. |
| 06 | With ANT 2 active via Nextion, switch to ANT 4 via the browser. Observe the Nextion. | Nextion button states update to show ANT 4 active. ANT 2 returns to grounded state. |  |  | If Nextion does not follow web commands — check syncButtonStates() call in controlRelay(). |

# 10. Antenna Name Storage (EEPROM)

| # | Action / Step | Expected Result | Pass | Fail | Notes / If Failed |
| --- | --- | --- | --- | --- | --- |
| 10.1 Rename via Web |  |  |  |  |  |
| 01 | Navigate to http://[IP address]/settings. Enter a distinctive name in each of the four antenna name fields (e.g. '20m Inv V', '40m Dipole', '80m Loop', 'Spare'). Click Save. | Page redirects. Nextion display updates immediately to show new antenna names above each button. Web interface shows new names. |  |  | If Nextion does not update — check EEPROM write and Nextion label push in /rename handler. |
| 10.2 Power Cycle Retention |  |  |  |  |  |
| 02 | Remove 12V power. Wait 10 seconds. Re-apply power. Wait for boot to complete. | Nextion displays the same antenna names entered in the previous step — names survived the power cycle. |  |  | If names reset to defaults — check EEPROM read in loadConfig(). Check EEPROM magic byte / version check. |
| 03 | Open web interface and navigate to /settings. Confirm names shown match those entered. | All four antenna names match — EEPROM consistent between Nextion and web. |  |  | If names differ — check that both read from the same config struct. |

# 11. Band Tracking & SO2R Interlock

This section verifies the /setband REST endpoint, Nextion band display, and SO2R interlock logic. Tests are performed via browser URL — no radio or Node-RED connection required.

| # | Action / Step | Expected Result | Pass | Fail | Notes / If Failed |
| --- | --- | --- | --- | --- | --- |
| 11.1 REST /setband Endpoint |  |  |  |  |  |
| 01 | In browser, navigate to http://[IP]/setband?input=1&band=40m | JSON response: {"input":1,"band":"40m","bandId":4,"so2r":false} |  |  | If 400 error — check input and band parameters. If band not recognised — check BANDS[] array in firmware. |
| 02 | Observe the Nextion display immediately after step 01. | tBandA updates to show '40m'. tBandB still shows '---'. tSO2R shows 'SO2R OK' in green. |  |  | If tBandA not updated — check updateNextionBandDisplay() is called in /setband handler. |
| 03 | In browser, navigate to http://[IP]/setband?input=2&band=20m | JSON response: {"input":2,"band":"20m","bandId":6,"so2r":false}. tBandB updates to '20m' on Nextion. |  |  | Check input=2 path in /setband handler. |
| 04 | Navigate to http://[IP]/status | Response includes: "bandA":"40m","bandB":"20m","so2r":0 |  |  | If band fields missing — check /status JSON construction in firmware. |
| 05 | Navigate to http://[IP]/setband?input=1&band=99m (unrecognised band name) | Returns HTTP 400 with {"error":"missing or unrecognised params"} |  |  | If accepts invalid band — check reverse lookup against BANDS[] array. |
| 11.2 SO2R Status Display |  |  |  |  |  |
| 06 | With tBandA=40m and tBandB=20m (different bands, not transmitting), observe tSO2R on Nextion. | tSO2R shows 'SO2R OK' in green. No inhibit active. |  |  | SO2R inhibit only fires when both ports are simultaneously transmitting. Check evaluateInterlock() logic. |
| 07 | Reset Input 2 band: navigate to http://[IP]/setband?input=2&band=40m (same band as Input 1). | tBandB updates to '40m'. tSO2R remains 'SO2R OK' — same-band inhibit only fires during simultaneous TX. |  |  | If tSO2R shows INHIBIT without TX active — check evaluateInterlock() early return condition. |
| 11.3 Band Reset |  |  |  |  |  |
| 08 | Navigate to http://[IP]/setband?input=1&band=160m and http://[IP]/setband?input=2&band=80m. Confirm Nextion updates correctly for both. | tBandA shows '160m', tBandB shows '80m'. tSO2R shows 'SO2R OK'. |  |  | Check both input paths in /setband handler. |

# 12. TCP Control Protocol (Port 9008)

This section tests the TCP control protocol using a terminal client. Use PuTTY (raw mode, port 9008), netcat, or similar. Commands are plain text lines prefixed C[seq]|

PuTTY setup: Connection type = Raw, Host = [IP address], Port = 9008. Disable local echo so you can see responses clearly.

| # | Action / Step | Expected Result | Pass | Fail | Notes / If Failed |
| --- | --- | --- | --- | --- | --- |
| 12.1 Connection & Greeting |  |  |  |  |  |
| 01 | Open a TCP connection to [IP address] port 9008. | ShackSwitch responds immediately with: S0¦device name=ShackSwitch ready=1 |  |  | If no response — check tcpServer.begin() in setup(). Check port 9008 is not firewalled. |
| 12.2 Ping |  |  |  |  |  |
| 02 | Send: C1¦ping | Response: R1¦00¦ |  |  | If no response — check processTcpCommand() for ping handler. |
| 12.3 Antenna List |  |  |  |  |  |
| 03 | Send: C2¦antenna list | Four response lines, one per antenna, each containing id=, name=, txBandMask=, rxBandMask=. Final line: R2¦00¦ |  |  | If names wrong — check myConfig.names[] array. Check EEPROM is loaded. |
| 12.4 Band List |  |  |  |  |  |
| 04 | Send: C3¦band list | 11 response lines covering 160m through 6m, each with id=, name=, freqStart=, freqStop=. Final line: R3¦00¦ |  |  | If wrong band count — check BAND_COUNT and BANDS[] array in firmware. |
| 12.5 Port Get |  |  |  |  |  |
| 05 | Send: C4¦port get 1 | Response includes: id=1 auto=1 band=[current] rxant=0 txant=0 tx=0 inhibited=0 |  |  | If invalid port error — check portById() function. |
| 12.6 Port Set |  |  |  |  |  |
| 06 | Send: C5¦port set 1 rxant=2 band=4 | Response: R5¦00¦ok followed by S0¦port id=1 status line. Relay 2 activates. tBandA on Nextion updates to '40m'. |  |  | If relay does not switch — check port set handler, rxant path for portId==1. |
| 12.7 Keepalive |  |  |  |  |  |
| 07 | Leave TCP connection open for 30 seconds without sending any command. | ShackSwitch sends S0¦ping after 30 seconds. |  |  | If no keepalive — check KEEPALIVE_INTERVAL and millis() timer in loop(). |

# 13. UDP Discovery Beacon

The ShackSwitch broadcasts a UDP discovery packet every 5 seconds on port 9008. This test uses a UDP listener on a PC on the same network.

On Linux/Mac: sudo tcpdump -i any udp port 9008. On Windows: Wireshark, filter: udp.port == 9008. Alternatively use a simple Python socket listener.

| # | Action / Step | Expected Result | Pass | Fail | Notes / If Failed |
| --- | --- | --- | --- | --- | --- |
| 13.1 Beacon Content |  |  |  |  |  |
| 01 | With ShackSwitch powered and connected to WiFi, run a UDP listener on port 9008 on a PC on the same network. Wait up to 10 seconds. | UDP packet received containing: SS name=ShackSwitch serial=SS-001 version=1.5.0 ip=[IP] port=9008 ant=4 radio=2 |  |  | If no packet received — check discoveryUDP.beginPacket() and endPacket() in handleDiscoveryBeacon(). Check broadcast address calculation. |
| 02 | Wait 10 seconds and confirm a second beacon is received. | Second beacon received within 5–6 seconds of the first. Beacon interval confirmed. |  |  | If interval is wrong — check BEACON_INTERVAL constant (should be 5000ms). |

# 14. Factory Reset

Performing a factory reset will clear all stored antenna names and WiFi credentials. Have your WiFi details ready to re-enter afterwards.

| # | Action / Step | Expected Result | Pass | Fail | Notes / If Failed |
| --- | --- | --- | --- | --- | --- |
| 14.1 Reset Procedure |  |  |  |  |  |
| 01 | On the Nextion, navigate to the config page. Press the Reset button once. | A warning message appears on the display prompting for confirmation. |  |  | If no warning appears — check the reset button trigger and warning dialog in HMI. |
| 02 | Wait 6 seconds without pressing Reset again (allow timeout to expire). | Warning disappears. No reset performed. Antenna names and WiFi still intact. |  |  | Confirms the double-tap safety timeout is working. |
| 03 | Press Reset button on config page, then immediately press Reset a second time within 5 seconds. | Arduino reboots. Nextion shows default antenna names (ANT 1–4). IP address field clears until WiFi reconnects. |  |  | If no reboot — check second press is received within timeout window. Check reset logic in firmware. |
| 04 | Navigate to /settings in browser after reconnect. Confirm names have returned to defaults. | All four antenna names show factory defaults. |  |  | If names not cleared — check EEPROM factory reset write in firmware. |

# 15. RF Path Test

Only perform this section after ALL previous sections have passed. Use maximum 5W for initial RF testing. Always connect a dummy load to any antenna port that does not have an antenna connected.

| # | Action / Step | Expected Result | Pass | Fail | Notes / If Failed |
| --- | --- | --- | --- | --- | --- |
| 15.1 Setup |  |  |  |  |  |
| 01 | Connect transceiver TX output → SWR/power meter → RADIO IN on ShackSwitch. Connect 50 ohm dummy load to ANT 1. Connect dummy loads (or short coax with terminator) to ANT 2, 3, 4. | All five SO239 ports connected. No open ports. |  |  | Open ports during TX will cause high SWR and potential damage. |
| 15.2 Each Port at Low Power |  |  |  |  |  |
| 02 | Select ANT 1 on Nextion. Key transceiver at 5W for 1 second. Observe SWR meter. | SWR reads 1.0:1 to 1.5:1 (dummy load). Power meter shows approximately 5W forward, minimal reflected. |  |  | High SWR — check coax connector seating. Check relay contact condition. |
| 03 | Select ANT 2. Key at 5W for 1 second. | SWR 1.0:1 to 1.5:1. Low reflected power. |  |  | High SWR on one port but not others — relay contact issue on that relay. |
| 04 | Select ANT 3. Key at 5W for 1 second. | SWR 1.0:1 to 1.5:1. Low reflected power. |  |  | See above. |
| 05 | Select ANT 4. Key at 5W for 1 second. | SWR 1.0:1 to 1.5:1. Low reflected power. |  |  | See above. |
| 15.3 Isolation Check |  |  |  |  |  |
| 06 | Select ANT 1. Connect an RF power meter to ANT 2 port. Key at 5W. | No significant power detected on ANT 2. Any reading should be >20dB below transmitted power. |  |  | If significant power on unselected port — relay not fully opening. Check grounding circuit. |

# 16. KK1L 2×6 Expansion Build (Future — Hardware Ordered)

Complete this section once the KK1L board and MCP23017 GPIO expanders have been installed. All sections 4–15 should be re-run after the expansion build, in addition to the tests below.

Do not attempt this section until hardware is in hand and sections 4–15 have passed on the base build.

| # | Action / Step | Expected Result | Pass | Fail | Notes / If Failed |
| --- | --- | --- | --- | --- | --- |
| 16.1 MCP23017 #1 — KK1L Control |  |  |  |  |  |
| 01 | With MCP23017 #1 fitted at address 0x20, scan the I2C bus at startup. Check Serial Monitor output. | Serial Monitor shows MCP23017 detected at 0x20. |  |  | If not detected — check I2C wiring (SDA/SCL), pull-up resistors, and address jumpers A0–A2. |
| 02 | Select Antenna 1 via Nextion. Probe MCP23017 #1 GPA1 pin with multimeter. | GPA1 HIGH (3.3V). GPA2–6, GPB1–6 all LOW. |  |  | If wrong pin HIGH — check MCP23017 bit mask in firmware. Pin number should equal port number. |
| 03 | Verify transistor/ULN2803 output for KK1L relay coil on Antenna 1. | 12V relay coil energised. Relay clicks. Continuity confirmed on KK1L ANT 1 output. |  |  | If no relay click — check transistor/ULN2803 driver stage. Check 12V supply to relay coils. |
| 04 | Repeat for all 6 outputs on Input 1 (GPA1–6). | Each antenna port switches cleanly. No more than one relay active at a time per input. |  |  | Check each driver transistor individually. |
| 05 | Repeat for all 6 outputs on Input 2 (GPB1–6). | Each antenna port switches cleanly on Input 2 bank. |  |  | Check GPB1–6 driver stage. |
| 16.2 MCP23017 #2 — Config Detection |  |  |  |  |  |
| 06 | With MCP23017 #2 fitted at address 0x21, set config jumpers for 2×6 board, 2 inputs, FlexRadio. Power on and check Serial Monitor. | Serial Monitor reports: Board=2x6, Inputs=2, RadioType=Flex, PA=0 (or as jumpered). Device operates in correct mode. |  |  | If defaults loaded instead — check I2C read of 0x21 in setup(). Check jumper connections. |
| 16.3 RF Path — KK1L |  |  |  |  |  |
| 07 | Connect transceiver to Input 1 SO239. Connect dummy loads to all 6 antenna outputs. Select each antenna 1–6 via Nextion and key at 5W. | SWR 1.0:1 to 1.5:1 on each of the 6 outputs. No simultaneous relay closure. |  |  | High SWR on specific port — check coax routing and relay contact on that output. |
| 08 | Repeat RF path test for Input 2. | SWR 1.0:1 to 1.5:1 on each of the 6 outputs for Input 2. |  |  | Check Input 2 relay bank and coax routing. |

# 17. Fault Diagnosis Quick Reference

| Symptom | Most Likely Cause | Action |
| --- | --- | --- |
| Display blank at power-on | 5V not reaching Nextion | Check buck converter output voltage. Check Nextion VCC and GND wiring. |
| Display shows garbled content | Wrong HMI file or TX/RX crossed | Re-upload HMI file via microSD. Swap TX1 and RX1 wires to Nextion. |
| N O T C O N N E C T E D on display | Wrong WiFi credentials | Edit SSID/password in firmware or via Nextion config page. Re-upload. |
| Web interface not loading | IP incorrect or firewall | Ping the IP address from browser PC. Check all devices on same subnet. |
| tBandA/tBandB not updating | /setband endpoint not reached or band name not recognised | Check /setband handler is in place. Check band name matches BANDS[] array (case sensitive). |
| tSO2R black or blank at boot | updateNextionBandDisplay() not called in setup() | Confirm updateNextionBandDisplay() is called after myNex.begin() in setup(). |
| TCP port 9008 not responding | tcpServer not started or port blocked | Check tcpServer.begin() in setup(). Check firewall on router for port 9008. |
| No UDP beacon received | discoveryUDP not sending or broadcast address wrong | Check broadcast address calculation. Check BEACON_INTERVAL timer. |
| Relay clicks but no continuity | Relay contacts faulty or wiring | Check RF wiring from relay NO contact to SO239. Check ground wiring. |
| Two relays active simultaneously | Firmware not enforcing single-antenna rule | Critical — re-upload firmware. Check relay control logic. |
| Antenna names reset on power cycle | EEPROM not writing or version mismatch | Check EEPROM magic byte in loadConfig(). Perform factory reset then re-enter names. |
| High SWR on all ports | RADIO IN not connected properly | Check coax from radio to RADIO IN SO239. Check centre pin continuity. |
| High SWR on one port only | Relay contact or wiring to that SO239 | Check relay NO terminal wiring to that specific SO239 port. |

# 18. Test Completion Sign-Off

Complete this section when all test steps above have been passed. Record any deviations or observations here for future reference.

| Builder callsign / name |  |
| --- | --- |
| Date completed |  |
| Firmware version | v1.5 |
| Hardware version / notes |  |
| Any deviations or observations |  |
| All sections passed — station ready for operation |  |

G0JKN ShackSwitch — open source under MIT Licence — github.com/nigelfenton/shackswitch